Main Feature

- AZ Relay Performs high reliability in dry circuit.

- High density design is available on P.C. Board with size of 10.4x15.4x11.2 in mm and the weight of 3.5 grams.

- The Employment of 2.54mm terminal pitch is equal to I.C.’s terminal pitch.

- The low power consumption of AZ-L type and general power consumption type of AZ-D are prepared for user’s wide selection.

- Complete protective construction is designed from dust and soldering flux.  In addition, the plastic sealed type is prepared for washing procedure.

Application

Telecommunication, Domestic Appliances, Office Machine, Audio Equipment, etc.

Contact Rating

- Nominal Load (Resistive Load Cosφ=1)

Contact Capacity	1A at	120VAC.

1A at	24	VDC.

Rated Carrying Current	1A.

Max. Allowable Current	1A.

Max. Allowable Voltage	AC 120V, DC 60V.

Max. Allowable Power Force130 VA, 30W.

Min. Switching Load	DC 1V, 1mA.

- Contact Material	Ag Alloy.

- Contact Form	SPST & SPDT.

Performance (at Initial Value)

- Contact Resistance	100m Max.@100mA,

6VDC

- Operate Time	AZ-D  5	mSec. Max.

AZ-L  10mSec. Max.

- Release Time	4 mSec. Max.

Dielectric Strength :

Between Coil & Contact	500VAC at 50/60 Hz

for one minute.

Between Contacts	500VAC at 50/60 Hz

for one minute.

- Surge Resistance	1,000V (between coil

& contact 1.2x50Sec.)

- Insulation Resistance	100 Mega  Min. at

500VDC.

- Max. On/Off Switching :

Electrical	30	Ops per Minute.

Mechanical	300	Ops per Minute.

- Temperature Range	-30	~	55C (D Type)

-30	~	75C (L	Type)

- Humidity Range	45~85% RH.

- Coil Temperature Rise	43C Max. (D Type)

25C Max. (L	Type)

- Vibration :

Endurance	10 to 55 Hz dual

amplitude width 1.5mm.

Error Operation	10 to 55 Hz dual

amplitude width 1.5mm.

- Shock :

Endurance 	500 m/S2 Min.

Error Operation	100 m/S2 Min.

- Life Expectancy :

Mechanical 	107  Operations at No

Load condition.

Electrical 	105  Operations at Rated

Resistive Load.

- Weight	About 3.5 g.

Safety Standard & Its File Number

- UL	E141060

- CSA	LR76598

Coil Specification (at 20C)

| Coil Sensitivity | Nominal Voltage (VDC) | Nominal Current (mA) | Coil Resistance (±10%) | Power Consumption (W) | Pull-In Voltage (VDC) | Drop-Out Voltage (VDC) | Maximum Allowable Voltage (VDC) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AZ-D | 3 | 150 | 20 | Abt. 0.45 | 70% Maximum | 5% Minimum | AZ 130% |
|  | 5 | 89 | 56 |  |  |  |  |
|  | 6 | 75 | 80 |  |  |  |  |
|  | 9 | 50 | 180 |  |  |  |  |
|  | 12 | 37.5 | 320 |  |  |  |  |
|  | 24 | 18.8 | 1,280 |  |  |  |  |
| AZ-L | 3 | 66.7 | 45 | Abt. 0.20 | 75% Maximum | 5% Minimum | 130% |
|  | 5 | 41.6 | 120 |  |  |  |  |
|  | 6 | 33.3 | 180 |  |  |  |  |
|  | 9 | 22.5 | 400 |  |  |  |  |
|  | 12 | 17 | 700 |  |  |  |  |
|  | 24 | 8.6 | 2,800 |  |  |  |  |

Ordering Information

| AZ | - | SS | - | 1 | 12 | ### D | M |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Contact Form: | Nil: One Form C M: One Form A |
|  |  |  |  |  |  |  |  | Coil Type: | D: Standard DC Coil L: High Sensitivity DC Coil |
|  |  |  |  |  |  |  |  | Coil Voltage: | 03: 3V, 05: 5V, 06: 6V, 09: 9V, 12: 12V, 24: 24V |
|  |  |  |  |  |  |  |  | Number of Pole: | 1: One Pole |
|  |  |  |  |  |  |  |  | Type of Sealing: | SS: Flow Solder Type SH: Plastic Sealed Type |
|  |  |  |  |  |  |  |  | Type: | AZ |

Classification

| Model | AZ |  |  |  |
| --- | --- | --- | --- | --- |
| Coil Sensitivity | Standard Type |  | High Sensitivity Type |  |
| Contact Form | 1C | 1A | 1C | 1A |
| Flow Solder Type | AZ-SS-1□□D | AZ-SS-1□□DM | AZ-SS-1□□L | AZ-SS-1□□LM |
| Plastic Sealed Type | AZ-SH-1□□D | AZ-SH-1□□DM | AZ-SH-1□□L | AZ-SH-1□□LM |

Dimension

Reference Data